VECTOR EQUINE  ·  ENGINEERING BRIEF

Phone as Remote

Running the Jetson headless, controlled entirely from the app

The target install: a Jetson and a camera. No monitor, no keyboard, no laptop in the aisle. The device is on the internet and does the heavy lifting on site; the rider's phone is the only interface; the finished ride lands in the rider's account in the cloud.

| Question | Answer |
| --- | --- |
| Is it possible? | Yes. The architecture already points this way. |
| How much exists? | The capture loop is built end to end: phone starts, device attaches, records to a shared clock, uploads, ride lands in the account. |
| What actually blocks it? | Three things: no command path down to the device, no way to get it on wifi without a keyboard, and pairing that requires typing on the device. |
| Biggest single unlock | Carry desired state on the heartbeat we already send. No new infrastructure. See §5. |
| Only genuinely new hardware work | Wifi onboarding over a device access point or Bluetooth. Everything else is app and agent work. |

Status

Proposal for team review — 28 August 2026. Nothing in §4–§6 is built yet. Section 2 describes code that is in main today.

# Contents

1 · What we are trying to build

2 · How it works today

3 · What the current design gets right

4 · The five gaps

5 · Recommendation: the heartbeat becomes the control plane

6 · Sequencing

7 · Decisions we need to make

Appendix A · Edge API surface as built

Appendix B · Key files

Appendix C · Explicitly not built

# 1 · What we are trying to build

A barn install that is nothing but the device and the camera. The rider walks up with a phone, taps once, rides, taps again, and the ride is in their account. No screen on the device, no keyboard, no service visit to configure anything.

This is achievable, and the foundation is already there. The phone already starts the session and owns the clock. The device already attaches to that session, records against that clock, and uploads into the rider's account. What is missing is narrower than it sounds, and only one item on the list is hardware work.

The phone can start a session today. What it cannot do is tell the device anything afterwards.

That is the crux. Every call in the current design is initiated by the device. There is no path from the cloud down to the Jetson, which means the phone can begin a lesson and then hope the device picked it up. Until that channel exists, “the phone works it” is not true in any meaningful sense. Section 5 proposes the cheapest credible fix.

# 2 · How it works today

This is the loop as built. No monitor is involved at any point in a lesson — the keyboard problem is entirely about first-time setup, not about riding.

| # | Step | Who | Mechanism |
| --- | --- | --- | --- |
| 1 | Rider taps Start | Phone | POST /api/capture/sessions creates the capture_sessions row and stamps t0 — the master clock everything else is measured against. |
| 2 | Device attaches | Jetson | POST /api/edge/sessions/attach with Authorization: Edge <device_key>:<device_secret>. Server finds that rider's most recent open session, binds the device, returns t0 and a 4-hour signed session token. |
| 3 | Device reports in | Jetson | POST /api/edge/sessions/{id}/heartbeat keeps edge_recording alive and updates edge_last_heartbeat_at. |
| 4 | Device uploads | Jetson | POST /api/edge/sessions/{id}/video (multipart) into the private session-videos bucket, with a row in session_media_assets carrying sync_offset_ms. |
| 5 | Device signals done | Jetson | POST /api/edge/sessions/{id}/complete returns the sync manifest. Deliberately does not end the lesson. |
| 6 | Rider taps End | Phone | POST /api/capture/sessions/{id}/end creates the durable training_sessions ride in the rider's account. |
| 7 | Debrief | Phone | Signed 6-hour playback URL; transcript timestamps scrub the player, because both sides share t0. |

Two properties of this loop are worth calling out because the rest of the plan depends on them. First, the phone is already the trigger — the device finds work by asking whether the rider has opened a session. Second, the cloud owns the clock, so video and transcript line up without the device and the phone ever talking to each other directly.

# 3 · What the current design gets right

The device makes outbound HTTPS calls only. This is the most valuable property we have and it should constrain every proposal that follows.

No inbound ports, no port forwarding, no static IP, no NAT traversal, no VPN.

Works unchanged on barn wifi, a phone hotspot, or an LTE modem — swap the network and nothing needs reconfiguring.

Nothing to change on the barn's router. This matters more than it looks: we will usually not have admin access to it, and often nobody on site will know the password.

Credentials are tightly scoped. A device secret only ever buys a session token bound to a single capture session, and it expires in four hours.

Design rule for review: any control-plane proposal that needs inbound connectivity to the device, or router configuration at the barn, should be rejected on these grounds alone.

# 4 · The five gaps

Three of these block a keyboard-free install. Two are about making it trustworthy enough to hand to a customer. Sizes are relative, not estimates in days.

| Gap | Blocks headless? | Where the work lives | Size |
| --- | --- | --- | --- |
| 1 · No cloud-to-device command path | Yes | This repo | S |
| 2 · Wifi onboarding | Yes | Device image | M |
| 3 · Pairing needs typing on the device | Yes | Repo + provisioning | M |
| 4 · Agent is a script, not a service | No — but it will bite | Repo + device image | M |
| 5 · Phone cannot see the device | No — trust problem | This repo | S |

## Gap 1 · There is no command path down to the device

### Today

Everything is device-initiated. attach even sets edge_recording = true unconditionally, so the device starts recording the moment it attaches rather than when the rider says so. There is no way to send stop, restart the camera, switch lens, re-upload, or reboot.

### Why it matters

This is the whole feature. Every other item on this list is quality of life; this one is the difference between a phone that opens a session and a phone that operates a device.

### Recommendation

Desired state on the heartbeat response — see §5. Relative size: small.

## Gap 2 · Wifi onboarding is the real keyboard

### Today

Nothing exists. A headless Jetson cannot learn the barn's network name and password without some form of input. This is the actual reason a keyboard is required today — not the app, not the pairing flow.

### Recommendation

On boot with no known network, the device raises its own access point (or advertises a Bluetooth service). The phone joins it once, hands over the credentials, and the device joins the barn network and drops the access point. This is a well-trodden pattern with several open-source implementations to start from, and it is the standard experience for every consumer camera and smart-home device on the market.

### Note for planning

This is the one item that needs embedded Linux work rather than app work, so it can proceed fully in parallel with everything else. Relative size: medium.

## Gap 3 · Pairing currently requires typing on the device

### Today

POST /api/edge/devices/pair returns a device_key and a one-time device_secret to the phone — and then a human has to get those onto the Jetson, which means SSH or a keyboard. The secret is shown once and never again.

### Recommendation

Invert the flow. Give each unit a factory identity plus a short code printed on the case, with a QR alongside it. The rider scans or types that code in the app; the device polls “have I been claimed?” until it receives its rider binding and a freshly rotated secret. The device never has to be told anything by a human.

The scanning pattern already exists in the app for trainer join codes and can be reused rather than invented. Relative size: medium, mostly because it touches how units are provisioned before they ship.

## Gap 4 · The agent is a one-shot script, not a service

### Today

The reference agent attaches, uploads one file, calls complete, and exits. If no session is open it exits on a 404. Boot the Jetson before the rider taps Start and it simply dies — which is exactly what will happen in a barn, where the device is powered on all day and the rider shows up at four.

### Recommendation

A long-running service under systemd with restart-on-failure, which:

idles in a wait-for-session loop instead of exiting when there is no work;

survives network drops and DNS failures without dying;

resumes in-flight uploads after a power cut;

rotates its session token before the four-hour expiry on long days.

Relative size: medium.

## Gap 5 · The phone cannot see the device

### Today

The data exists — edge_recording and edge_last_heartbeat_at on capture_sessions — but no phone-facing endpoint surfaces device health and no screen renders it. Live camera preview is explicitly out of scope in the current edge doc.

### Why it matters

A rider mounts a camera, walks out to the arena, and has no way to know whether anything is capturing. They will find out an hour later, when the ride is gone. Confidence in the box is the product.

### Recommendation

A small device-status read on the session and one calm status line on the live screen: ready, capturing, last seen a moment ago. It states facts and draws no conclusions, in keeping with the measurement voice. Preview can wait; presence cannot. Relative size: small.

# 5 · Recommendation: the heartbeat becomes the control plane

We already make a round trip to the device on a timer. The cheapest control plane is to stop wasting the return half of it.

Today the heartbeat is a one-way report: the device says “I am recording,” the server says “ok.” Change the response so it also carries desired state, and let the device reconcile the difference.

POST /api/edge/sessions/{id}/heartbeat { "recording": true, "layers": { "video": true, "sensors": false } } 200 OK { "ok": true, "t0": "2026-08-28T14:02:11.480Z", "status": "live", "desired": { "recording": true, "video": true, "sensors": false, "upload": "after_end" }, "commands": [ { "id": "c_8f2a", "op": "restart_camera" } ], "heartbeat_ms": 2000 }

The device compares desired against actual, acts on the difference, and acknowledges command ids on its next beat so nothing runs twice. The phone writes desired state through an existing authenticated route; the server persists it on the capture session.

## Why this over the alternatives

No new infrastructure. No new dependency, no new service, no inbound connectivity. This ships in days, not weeks, and it does not put anything else at risk.

Latency is just the heartbeat interval. Run it at one to two seconds while a session is open and back off to thirty when idle. A rider tapping stop and seeing it land within two seconds is indistinguishable from instant.

Crash-safe by construction. Desired state is the truth, so a device that reboots mid-lesson recovers by asking once. A plain command queue would not give us that — it would give us a device that missed the message and never knows.

Tunable from the server. Because the server returns heartbeat_ms, we can change responsiveness in production without shipping a device update. That is worth a lot when the fleet is in barns.

## Alternatives considered

| Option | Assessment |
| --- | --- |
| Supabase Realtime | True push instead of polling. But there is no Realtime usage anywhere in the codebase today, so it is all new plumbing, plus a persistent socket to keep alive on a device that may be on flaky barn wifi. Revisit only if sub-second control ever genuinely matters. |
| Device joins the live room as a participant | Heavier, but the right answer the day we want live camera preview on the phone, since the room and the token minting already exist. Recommend deferring until preview is actually on the roadmap, then doing it properly rather than half-doing it now. |
| Inbound connection to the device | Rejected. Breaks every property in §3 and would require router access we will not have. |

# 6 · Sequencing

Phases A and B are independent enough to run in parallel if the embedded work has a separate owner. Nothing here requires touching auth, payments, or the social plumbing.

| Phase | Work | What it gets us |
| --- | --- | --- |
| A · Make the phone the remote | Desired state on the heartbeat and a reconcile loop in the agent · stop setting edge_recording on attach so the phone commands record · device status read plus a status line on the live screen | The rider controls capture from the phone. Still needs a keyboard once, at install. |
| B · Remove the keyboard | Wifi onboarding via device access point or Bluetooth · claim-code and QR pairing with the device polling for its binding · systemd service, wait-for-session loop, token rotation, resumable state | A box and a camera. Nothing else in the aisle. |
| C · Survive a real ride | Chunked or resumable upload using signed URLs direct to storage · upload after End so barn wifi never gates the lesson · measured clock skew at attach and time sync on the device | Multi-hour rides upload reliably and stay in sync with the transcript. |
| D · Move the heavy lifting to the edge | On-device inference · upload a compressed proxy plus derived numbers instead of raw footage · extend the media asset kinds beyond video, sensor, and audio | The edge device earns its name, and upload cost and time collapse. |

# 7 · Decisions we need to make

Both of these want an answer before Phase C starts, because they change the contract.

## Where the heavy lifting actually happens

Today the Jetson only records and uploads. Transcript, speech, and the ride journal all run in the cloud. If the intent is real inference on the device, then the upload contract has to carry derived data, and the media asset kinds currently permit only video, sensor, and audio.

The payoff is large. Keep raw footage local and ship a compressed proxy plus the extracted numbers, and we sidestep the current ceiling entirely: the video route is a single multipart upload that rejects anything over roughly 95 MB, and a real ride is gigabytes.

## Clock precision

Everything stamps an offset against a cloud-owned t0, and the reference agent currently hardcodes that offset to zero. That is fine for a demo and not fine for a product that reports release timing in tenths of a second.

The device needs proper time sync plus a skew measurement captured at attach. Without it, video and transcript drift apart in exactly the measurements that matter most to us — the ones about when an aid was given and when it was released.

A tenth of a second of drift is the difference between a read and a guess.

# Appendix A · Edge API surface as built

| Route | Method | Auth | Purpose |
| --- | --- | --- | --- |
| /api/edge/devices/pair | POST | Rider session | Pair a device. Returns device_key and a one-time device_secret. |
| /api/edge/devices/pair | GET | Rider session | List the rider's paired devices, no secrets. |
| /api/edge/sessions/attach | POST | Edge key:secret | Bind to the rider's open session. Returns t0, a 4-hour session token, and the manifest. |
| /api/edge/sessions/{id}/heartbeat | POST | Bearer edge token | Keep recording state alive and report which layers are running. |
| /api/edge/sessions/{id}/video | POST | Bearer edge token | Multipart upload into the private videos bucket. Rejects over ~95 MB. |
| /api/edge/sessions/{id}/complete | POST | Bearer edge token | Signal upload finished and return the sync manifest. Does not end the lesson. |

# Appendix B · Key files

| Path | What it is |
| --- | --- |
| docs/edge-sync.md | The current architecture note — phases 0 to 2 and what is out of scope. |
| edge/jetson-agent/agent.py | Reference agent. The one-shot script described in Gap 4. |
| edge/jetson-agent/README.md | Pairing and run instructions, plus the API contract. |
| src/app/api/edge/** | All five edge routes. |
| src/lib/capture/edge-token.ts | Device secret hashing, signed session tokens, manifest builder. |
| src/lib/capture/edge-auth.ts | Device and session authentication helpers. |
| supabase/migrations/20260804000000_edge_devices.sql | The devices table and the edge columns on capture sessions. |
| src/components/capture/lab-edge-pair.tsx | Lab pairing UI that surfaces the one-time secret. |
| src/components/capture/capture-room.tsx | Live session client, including the QR pattern worth reusing for pairing. |

# Appendix C · Explicitly not built

Listed so nobody plans around it. None of the following exists in any form today.

Any command or message from the cloud to the device.

Wifi onboarding, or any way for the device to learn a network without a keyboard.

Pairing that does not require getting a secret onto the device by hand.

A supervised, always-running agent that waits for work.

Device health or presence anywhere in the app UI.

Live camera preview on the phone.

Chunked or resumable upload for long rides.

Sensor ingest — flagged as a later phase, and the sensor layer is a stub with its flag off.

On-device inference of any kind.

Clock skew measurement between the device and the platform clock.

Firmware or agent update delivery to a fleet.

Vector Equine · Phone as Remote · Proposal for review, 28 August 2026. Sections 4 through 7 describe work not yet started.